PnPKIT V2.10

Copyright (c) 1996-1999, Sekhar Bala,

Ramchander Balasubramanian, and

Alphax Systems, Inc.

1.0 - Introduction

PnPKit is an intelligent script language processor that allows

users to experment, rectify, and query the Plug-And-Play settings

of any/all pc-boards in a machine that implements the standard

Plug-And-Play Specifications.  If your an experience designer/developer,

more commonly called the 'Plug-And-Pray' Specification.

2.0 - Build

Unload the archive into an appropriate subdirectory structure.  From

the Root directory do the following:

make -f Makefile.<os>

The .../bin directory contains the executables.  The build process

requires a GNU-like capable 'make' ; namely, the 'include' and

conditional macros features are needed.  With some minor modifications

the system should port to any environment.  The primary environment of

the build process is under Linux with GNU C++.

3.0 - Language Specific Usage

A. General Commands

Each line entry constitutes a single command.  All keywords are

case insensitive (all chars are treated as uppercase).

#                            + are treated as comment excluders

all text following the pound-char

are ignored by the parser.  Also,

semicolons ';' can be used to

terminate a line.

echo [on | off]              + if 'on' then, all functions &

executed scripts will display

each execution command.  If 'off',

performs silent exection.  If

neither 'on' or 'off' is specified,

the current echo state is displayed.

quote [on | off]             + make string '$' var references have

or omit quotations, respectively.

print [opts] user-text       + displays user-text to the display.

[opts] are:

-n  - suppress newline

-e  - translate user-text

@filename[.XXX]              + redirect execution to the specified

script file.  The default filename

extension is system specific.

return                       + return from script file or function

exit | quit                  + leave system

B. Declarations & Definitions

All variables and functions take global scope (there are no

local variables and/or functions with the context of a

script or function-level).

Variables are referenced by name with a prefix-character of '$'.

The following lists the declaration syntaxes:

INT   varname1         + declares a 16-bit integer named varname1

INT32 varname2         + declares a 32-bit integer named varname2

CHAR  varname3         + declares an 8-bit char    named varname3

CHAR  varname4[n]      + declares a string of length n named varname4

The following commands describe system operations on variables:

SHOW  [hex] varname    + displays system information about the varname

DELETE      varname    + removes the declared variable from the system

Functions can be declared and defined with the following

sequence of characters.

FUNCTION fname1( )

{

any-set of emulatemux cmds

}

FUNCTION fname2( 0 )

{

any-set of emulatemux cmds

}

FUNCTION fname3( 2 )

{

any-set of emulatemux cmds

}

'fname1' defines a function with no arguments.  'fname2' also

defines a function with no arguments.  'fname3' defines a

function with 2 arguments.  Either argument within the 'fname3'

function-body is referenced via variable context $1 or $2.  In

general $1, .. $n as specified with the '( )' of the function

declaration.

Functions are invoked with the following syntax

call fname( arg1, arg2, ... )

Note: echo state shall display command execution during function

invocation.

C. Expressions

Expressions within the system take two forms:  expr

and sexpr processing numerical and string manipulations,

respectively.  expr syntax is as follows:

var = expr          + set numerical variable value

+ No precedences take effect on operators;

expr must distinquish terms with higher

precendences by explicitly using '()'

parenthesis

+ left of '='

1- cannot contain '$' prefix

2- can contain var[X] in string array

where X is not an expr

+ right of '=' can contain

1- as expected operator (+,-,*, etc...)

1- '$' prefix variables

2- single string indexed positions:

$var[X]  where X is a const or $VAR

and X cannot be an expr

3- reference to any built in variable:

$NOW  <- returns current date/time in binary

var = sexpr         + set string variable value

+ right of '=' can contain

1- + operator for STRING + STRING

2- built-in string functions

$DT_DATE( [dt#] )     - MMM-DD-YY

$DT_DATEY2( [dt#] )   - same as above

$DT_DATEY4( [dt#] )   - MMM, DD YYYY

$DT_TIME( [dt$] )     - hh:mm:ss

$DT_DATETIME( [dt#] ) - MMM, DD YYYY hh:mm:ss

$SYS_ENV( "envvar" )  - return shell envvar

D. Control Statements

The following control statements are processed:

while (expr) {

...

break

...

continue | cont

...

}

for( stmt1 [, stmt2, ...]; (expr) ; stmta [, stmtb, ...] ) {

...

break

...

continue | cont

...

}

if ( expr ) {

...

}

if ( expr ) {

...

} else {

...

}

if ( expr ) {

...

} else if ( expr ) {

...

}

E. PnP related Commands

isolate [arg]             + peform card isolation

args are:

CSN       - reset each card's csn (*default)

RESET     - set cards to factory settings

RESET_DRV - set cards to power-up defaults

detect <csn-#>            + select/identify card for manipulation

make csn-# the current card

status <csn-#>            + show status of current cfg

dump [args]               + dump/display current card information

args are:

RESOURCE   - card's avail resources

DEVICE     - card's devices summaries

DEVICE ALL - card's devices verbose

CARD       - card's header information

device <action> [values]  + set device attribute

actions are:

BEGIN - initiate device cfg action

END   - end and perform device cfg actions

SET   - see description below

* device SET <resource> <values>

resources are:

MEM, MEM32, IOPORT, IRQ, DMA, ACTIVE, INACTIVE

values are:

CLEAR or specific value/tag pairs specified below

MEM  n #1 #2 [b]

- set memory mapped region n (0 - 3)

#1 = base address bits 23-8

#2 = length bits 23-7

b  = kw 8bit(*) or 16bit for mem-type

MEM32 n #1 #2\n"

- set memory mapped usage

IOPORT  # [, # ...]

- set ioport resources

IRQ n  #  [HIGH | LOW(*)]  [LEVEL | EDGE(*)]

- set IRQ n (0 or 1) with #

DMA n  #

- set DMA n (0 or 1) #

ACTIVE or INACTIVE

- activate or deactivate card\n"

* device SET <resource> CLEAR

- completely clear resource value ( eg 'device SET IRQ CLEAR' )

---