/*

---------------------------------------------------------------

Roland MPU-401 MIDI driver for Linux

---------------------------------------------------------------

This is a somewhat quick hack to get Rolands MPU running under

Linux. The driver is not compatible to the existing drivers

for soundblaster etc. because the mpu-hardware is quite different

from (what i know about) soundblaster, eg. mpu has it's own

timer, can synchronize via MIDI, 'knows' about MIDI message

formats etc.

So, if you want to write an application for it, it wouldn't

run with soundblaster & friends.

This version of the driver does not support mpu completely:

- conductor is not supported, eg. If an application wants

to send mpu-specific commands (e.g. change speed), this

cannot be done by putting the command into the interrupt

queue. Speedchange (and others) are executed immediately

when received by the driver.

- only MIDI voice messages can be recorded (e.g. Note on/off,

pitch, controller etc). System exclusive, realtime messages

(e.g. song pointer) are not supported yet.

This is the bad news, now the good ones: it works and programming

is quite easy, but you'll need a mpu-401 reference manual.

Midi-Output: you have to write complete commands to the driver, if a

command is 10 bytes long you can't write() 5 bytes now and the other 5

bytes later. This is why 'cat file > /dev/mpu401' will not work.

A command starts with a byte, that tells the driver the length

of the mpu-data and where to put it.

Bits 0..5: length of MPU-Data following.

Bits 6..7:

00 : send later on Track Data Request Interrupt

(0xF0-0xF6, 7 Tracks supported, track number follows leading byte)

01 : write immediately to Data Register

10 : write immediately to Command Register

11 : driver command: 0xc0 = reset everything

#define TRK (0<<6)

#define DAT (1<<6)

#define CMD (2<<6)

#define RES (3<<6)

for an example see ../../test/mpustress.c